Annex A

Frequency Allocation

for In-Building Coaxial Cable Systems

Homes Passed

- Communal Aerial Broadcast Distribution (CABD) systems have been installed in most of high-rise buildings in Hong Kong.  These systems are not required to be licensed.

- As at 31 May 1998, about 550,000 households were connected to about 1,600 Satellite Master Antenna Television (SMATV) systems.

- Wharf Cable has passed approximately 1.6 million households as at 31 May 1998.

VHF Spectrum (54-470 MHz)

- At present, cable television (CATV) service operates in the band of 54-470 MHz.  Apart from channels that are prohibited by the Telecommunications Authority to be used (to avoid interference to essential services), there are 41 usable channels in the band of 54-470 MHz.  At present, 40 channels are being used for CATV service (9 of which are subject to renewal by the TA) and half a channel is allocated for cable telephony service.

- The transmission plan of the CATV service is fixed (i.e. the same plan throughout the territory).

UHF Spectrum (470-790 MHz)

- The CATV systems and most of the SMATV/CABD systems are designed to operate up to 750 MHz (some up to 790 MHz).

- 3 channels (in the band of 766-790 MHz) have been allocated for CATV set-top converter output.

- Closed Circuit TV system usually occupies one channel in Ultra High Frequency (UHF) Band.

- One channel at least has to be reserved for video-on-demand (VOD) output.

- One channel has to be reserved for video cassette recorder (VCR) output.

- Terrestrial TV occupies 4 channels in the UHF Band.  However, different terrestrial TV transposers use different groups of transmitting frequencies.

- Depending on the frequency group used for terrestrial TV at certain locations, SMATV operators will choose vacant channels in the band of 470-750 MHz to distribute the SMATV signals.  Most SMATV systems distribute 5 satellite TV channels but some may distribute more than 20 channels.  Therefore, the frequency plan in UHF band cannot be fixed.

- The numbers of channels in the bands of 470-750 MHz and 750-790 MHz are 35 and 5 respectively.  At present, the number of vacant channels in the band of 470-750 MHz is normally about 8-23, depending mainly on the number of SMATV channels being received.